Instruction manual of MT26
6 in 1 petrol multi-purpose tools
[image: image86.jpg]



[image: image1.jpg]




[image: image72.jpg]geprifte
Sicherheit




wuxi hushan plant protection machinery factory
Yangwan village,hudai town,binhu district,wuxi city,P.R.china.
[image: image2.jpg]Before putting the unit in operation, read this instruction
manual thoroughly .




List of Contents

Description                                                                              Page 
1. Unit...........................................................................................................................................................................4


� 1.1 Scope of Delivery......................................................................................................................................4


� 1.1.1  6 in 1 Driving Unit...................................................................................................................................4


� 1.1.2  6 in1 Extension.......................................................................................................................................5


� 1.1.3  6 in 1 Pruning saw..................................................................................................................................5


� 1.1.4  6 in 1 Hedge trimmer ..............................................................................................................................5


� 1.1.5  6 in 1 blower............................................................................................................................................5
.
� 1.1.6  6 in1 Brush cutter.....................................................................................................................................6


� 1.1.7  6 in1 grass trimmer....................................................................................................................................6
 1.1.8  6 in1 edger............................................................................................................................................... 6
� 1.2 Warranty ........................................................................................................................................................6


2.  General Safety Instructions .....................................................................................................................................7


� 2.1 Emergency Action......................................................................................................................................8


� 2.2 Signs on Unit..............................................................................................................................................8


� 2.3 Assigned Use.............................................................................................................................................9


� 2.4 Residual Hazards and Protective Action....................................................................................................10


� 2.4.1 Mechanical Residual Hazards.................................................................................................................10


� 2.4.2 Human Factor Neglect ..........................................................................................................................10


� 2.4.3 Residual Thermal Risks ........................................................................................................................10


� 2.4.4 Noise Risks ...........................................................................................................................................10


� 2.4.5 Material and Other Substances Action Risk..........................................................................................10


� 2.4.6 Vibration Risk........................................................................................................................................10


� 2.4.7 Other Hazards.......................................................................................................................................11


� 2.4.8 Disposal ................................................................................................................................................11


� 2.5 Operator Requirements............................................................................................................................11


� 2.5.1 Qualification ..........................................................................................................................................11


� 2.5.2 Minimum Age ........................................................................................................................................11


� 2.5.3 Training..................................................................................................................................................11


3 .  Technical Data.......................................................................................................................................................11


4.   Transportation and Storage ...............................................................................................................................12

5.   Assembly and Initial Operation ...........................................................................................................................12
..
� 5.1 Set of Petrol multi-purpose tools 6 in 1: Driving Unit...................................................................................12


� 5.2 Set of Petrol multi-purpose tools 6 in 1: grass trimmer................................................................................13


� 5.3 Set of Petrol multi-purpose tools 6 in1: Brush cutter…………………………………........................................13


� 5.4 Set of Petrol multi-purpose tools 6 in 1: blower……………………………………….........................................13


� 5.5 Set of EngineGardenTools5in1:Hedge trimmer …………………………………………....................................14


� 5.6 Set of Petrol multi-purpose tools 6 in 1: Pruning saw ………………………………………………....................14
        5.7 Set of Petrol multi-purpose tools 6 in 1:.. Extension........................................................................................15


� 5.8 Set of Petrol multi-purpose tools 6 in 1: edger ................................................................................................15


� 5.9 Shoulder harness Assembly............................................................................................................................15


� 5.10 Fuel Information.............................................................................................................................................15


� 5.11 Initial Operation Safety Instructions...............................................................................................................15


6   Operation.....................................................................................................................................................................16


� 6.1 Starting 6 in1 multi-purpose tools....................................................................................................................16


� 6.2 Blade Removal................................................................................................................................................18


� 6.3 Ignition plug replacement ................................................................................................................................18


� 6.4 Operator Safety Instructions ............................................................................................................................19


�    6.4.1 grass trimmer and Brush cutter Operation Instructions.............................................................................19


�    6.4.2 Pruning Saw Operation Instructions..........................................................................................................20


�    6.4.3 blower Operation Instructions ...................................................................................................................20


�    6.4.4 Hedge trimmer Operating Instructions.......................................................................................................20
�    6.4.5 Edger Operating Instructions.....................................................................................................................20
� 6.5 Step-by-Step Instructions...................................................................................................................................21


7    Troubles-Causes-Troubleshooting ...........................................................................................................................21


8    Inspections and Maintenance ....................................................................................................................................22
..
� 8.1 Inspections and Maintenance Safety Instructions ..............................................................................................22


� 8.2 Inspections and Maintenance Schedule..............................................................................................................22


9    Spare Parts....................................................................................................................................................................23
.
Every effort is made to improve our products continuously. Therefore, technical data and figures may change. 

1.  Unit


	1.1 
	Scope of Delivery

	[image: image3.emf]
	All the incorporated tools, except for the engine driving unit have been factory pre-assembled 

	1.1.1 
	6 in 1 Driving Unit 


A universal combination set enabling perfect care of the garden and vegetation. Just one instrument is suffent to mow the lawn, cut the hedgerow and lop the trees. 

1. Handle 
2. harness loop hole 
3. stop switch 
4. 
[image: image4.emf]4. throttle lockout
5. Air filter 
6. Start handle
7. Fuel tank 
8. Throttle trigger
9. [image: image73.emf]Shoulder harness 
[image: image74.jpg]


10.Unit attachment locking screw 
11.throttle lock 
12.blade
13.Fixing screws with spring rings 
14.Nut 
15.Blade locking nut 
16.Blade protection 
17.Bracket diameter flange 
18.Spacer disc 

Fig. 1

[image: image75.jpg]1
8 xJ\m



1.1.2  6 in1 Extension 

Fig. 2

1. Extension

2.  Locking nut 

3.  Incorporated tools screwed connection 

1.1.3  6 in 1 Pruning saw 

[image: image5.jpg]


 Fig. 3 
1 Lock screw 

2 Automatic oil pump 

3 Oregon chain 

4 Oregon quality blade 

1.1.4  6 in 1 Hedge trimmer 

1 Lock screw 

2 Cutting unit adjusting lever 180° 

3 Adjusting lever lock 

4 Cutting unit 

[image: image76.jpg]


[image: image77.jpg]


[image: image78.jpg]


[image: image6.jpg]Lock screw

Cutting unit adjusting lever
180°

Adjusting lever lock
Cutting unit





1.1.5  6 in 1 blower:
1.  Lock screw 

2. blower unit. 

[image: image79.jpg]



[image: image80.emf]Fig. 5

1.1.6  6 in1 brush cutter:

[image: image7.jpg]Fig. 6

1. Lock screw
2. Blade protection
3. Shearing blade






1.1.7  6 in 1 grass trimmer:

[image: image8.jpg]1. Lock screw
1 2. String protection
2 3. String coil
3

Fia 7





[image: image81.emf][image: image82.emf]1.1.8  6 in 1 edger:

[image: image83.emf][image: image84.emf][image: image85.jpg]


[image: image9.jpg]





Fig 8
1.2 Warranty 

Warranty claims according to the warranty certificate attached

2. General Safety Instructions:
Read the instructions for use thoroughly before the initial operation of the machine. If any doubts regarding the connection and operation arise, consult the manufacturer (servicing department). 

Observe the instructions shown below carefully to guarantee the high degree of safety. 
Caution! 
To be safe at working with this brush cutter, it is required that instructions and information on safety, installation and operation contained in the manual are precisely observed. Any person engaged in the operation or maintenance of the machine should be knowledgeable regarding the operation manual and aware of any potential risk. Prevent children, the sick and infirm from access. While you are in the working area, look after the children very carefully. Observe any regional and local safety precautions applicable in your place. The same applies to any provisions on the safety at work and health protection in the work site. 

The manufacturer shall not be held liable if the machines it manufactures are altered in an unauthorised manner, in result of which damage was inflicted. 

Warning! It is necessary to observe the essential safety precautions when using the machine tools. Kindly follow the tips and instructions in the additional safety precautions. 

1. Pay attention to the conditions of the area, which you’re working in. After the engine is started, the machine will produce poisonous flue gas. The gas may be invisible and odourless. Hence never work with the machine closed or poorly ventilated areas. Provide for good lighting when cutting. Provide for stability when working in wet, snow or icy weather conditions on slopes or rough ground. 

2. Do not let any unauthorised persons approach the machine. The visitors and viewers, primarily the children, the ill and infirm persons should not have any access to the working area. Prevent other persons from any contact with the tools. 

3. Make provision for the tools storage. The machines not in current use should be kept in a dry place, elevated or locked where practicable so that any access to them is prevented. 

4. Always use a correct tool for the job to be done. E.g. avoid using small tools and accessories for work that should be actually done with a heavy tool. Use the tools only for the purposes they were designed for. 

5. Pay attention to adequate clothing. The clothing should fit the purpose and should not put any limits to you when working. Wear clothing with stiffener protecting against cuts. 

6. Use personal protection equipment. Wear safety shoes with steel toes/ steel or rough sole. 

7. Wear goggles. The objects may be thrown in your face. 

8. Wear ear protectors. Wear personal protective equipment to protect your ears, such as ear protectors. 

9. Hand protection. Wear hard gloves – gloves of leather give a good protection. 

10. Brush cutter operation. Never work with the cutting tool unprotected. Danger of injury with thrown-off objects. 

11. Remove socket wrench. Before switching the scythe on, remove all the wrenches etc. 

12 Be attentive any time. Mind what you are doing. Use common sense. Do not use the scythe when tired. Never work with the brush cutter when influenced by alcohol, drugs or medicaments affecting the concentration capability. 

13 Fuelling. Always switch the engine off before filling up. Open the tank cap carefully so that any overpressure may be let out and the fuel does not splash out High temperatures build up on the machine body when working with the scythe. Therefore, leave the machine to cool down before filling up the fuel. Otherwise, the fuel could ignite spontaneously and there is a risk of personal burns. Replenishing the fuel, mind not to overfill the tank. If the fuel drips off, it should be removed immediately and the machine cleaned. When the filling is completed, check the screw After filling up, check screw closure for the firm fit to prevent loosening at work because of vibrations. 

14 Time of Use and Breaks for Rest . After using the power machine for a longer time, you may have troubles with the capillary return in hands. However, you may prolong the time of use when wearing suitable gloves and taking regular breaks. Remember that the time of using the power machine may get shorter in result of personal inclination to poor capillary return, low outside temperature or strong contact force of the machine while working 

15 Watch out for any damaged parts. Inspect the machine before use. Are any parts damaged? Ask yourself seriously when detecting slight damage whether the machine will work perfectly and safely despite that. Watch out for proper aligning and adjustment of any moving parts. Do the parts fit one in another properly? Are the part damaged? Is everything installed as appropriate? Are any other prerequisites of proper working in place? Damaged protective devices etc. must be repaired or replaced by authorised entities unless the operation manual expressly say otherwise. Any defective switches must be replaced at an authorised servicing centre. Do not use the machine if it cannot be put on and off with the switch. When repairs are necessary, kindly turn to your local servicing centre. 

16.  Always put the engine off prior to any adjustment or maintenance of the machine. The same applies to any works on the mowing head. 

17.  Use approved parts only. Use only identical spare parts for maintenance and repairs. Request advice of an authorised servicing centre for this purpose. 
Warning! Use of other mowing head, accessories and superstructure parts that are expressly recommended may result in a threat to persons and property. . 

The machine should not be used for other than intended purposes. Any deviation from the purpose shall be considered an unauthorised use. Only the user, and never the manufacturer, shall be held liable for any material and personal damage resulting from such an unauthorised use. 

The manufacturer shall not perform any warranties if the machines manufactured by it are altered or used in an unauthorised manner and if damage is inflicted in result. 

CAUTION! 
Even if the machine is professionally used, there is always residual risk that is impossible to exclude. It is the type and design of the machine, which potential risks may be derived from as follows: 
· Contact with uncovered mowing head (laceration). 

· Reaching the running scythe (laceration) . 

· Hearing impairment, if proper ear protection is not used . 

· Dust/gas formation harmful to the health if the machined is used in confined areas (nausea). 
· Must assemble the extension when use the pruning saw and hedge trimmer。
2.1 Emergency Action

Apply the first aid adequate to the injury and get qualified medical assistance as quickly as possible. Protect the injured person from more accidents and calm him/her down. 

2.2 Signs on Unit 

Meaning of Symbols 
Symbols shown below are used throughout this manual and/or on the unit : 

Product Safety 
[image: image10.jpg]



Product compliance with respective EU standards 

Bans: 
	[image: image11.jpg]




	No fire, no unprotected source of light, no smoking 


Warning 
	[image: image12.jpg]



	[image: image13.jpg]A




	[image: image14.jpg]



	[image: image15.jpg]



	
[image: image16.emf][image: image17.jpg]



	[image: image18.jpg]i-R






	Warning/caution 
	Beware of ground obstacle
	Beware of faling objects 
	Keep at safe distance 
	Beware if health hazardous gases 
	Keep bystanders away




	[image: image19.jpg]e (&)





	[image: image20.jpg]



	[image: image21.jpg]



	[image: image22.jpg]



	[image: image23.jpg]



	[image: image24.jpg]"o . &






	guards designed to be opened shall have a sign warning of the relevant hazard visible on

the machine both when the guard is closed and when it is opened or removed;


	Keep in mind that the cutting head cannot come into contact with alien matter at its start and in the working operation.

 
	Keep at least 15 m as safe distance from any third person when working.


	Cutting blade: maximum allowable rotate speed
	Thrown object
	Keep bystanders at a safe distance.


	[image: image25.jpg]



	[image: image26.jpg]



	[image: image27.jpg]8500 min”






	Direction of edger blade rotation.

	Use string only for the work. Metal cutting blade is forbidden 
	Cutting string: maximum allowable rotate speed 


Commands: 
	[image: image28.jpg]



	[image: image29.jpg]



	[image: image30.jpg]



	[image: image31.jpg]



	[image: image32.jpg]



	[image: image33.jpg]




	Use ear protectors 
	Wear protection shoes 
	Use protection gloves 
	Wear protection clothing 
	Use face shield t 
	Read operating manual before use 


Environment Protection 
	[image: image34.jpg]



	[image: image35.jpg]&




	[image: image36.jpg]



	[image: image37.emf]

	Wastes to be disposed of in a professional manner not to harm the environment . 
	Cardboard packaging to be collected for recycling . 
	Faulty and/or disposed of electrical/electronic appliances to be collected by authorised salvage places . 
	Green Dot –Duales System Deutschland AG 


Packaging: 

	[image: image38.jpg]



	[image: image39.emf]
[image: image40.emf]
	[image: image41.jpg]




	Protect from moisture 
	Keep Up 
	Fragile 


Technical Data : 
	
[image: image42.emf]

EMBED Photoshop.Image.7 \s[image: image43.emf]

EMBED Photoshop.Image.7 \s[image: image44.emf][image: image45.jpg]



	[image: image46.jpg]



	[image: image47.jpg]



	[image: image48.jpg]



	[image: image49.jpg]



	[image: image50.jpg]




	Capacity 
	Engine output 
	Rpm 
	Fuel 
	Weight 
	Acoustic power level 


Product specific data: 
	[image: image51.jpg]




	Mixing ratio 


2.3 Assigned Use 

The unit is designed for general use in agriculture and forestry, gardening and agricultural, forest and garden areas treatment . 

The manufacturer shall not be made liable for damages if general rules and this manual regulations are not observed. 

2.4 Residual Hazards and Protective Action

2.4.1 Mechanical Residual Hazards 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Cutting, incision 
	String and blade may cause severe incisions . 
	Watch out for persons and animals not to stay in the work area. 
	

	Catching, winding 
	Wires and cords may be caught by the blade or the string coil and may both damage the machine and cause injury. . 
	Remove any foreign objects from the land to be mown prior to getting down to work. 
	dnager

	Stroke 
	Strong back flashes may occur on the blade working thick shrubbery . If the blade gets stuck, back flashes may occur in the extension tube. 
	Do not work on thick shrubbery with the machine . Cut at sufficient speed or avoid jamming of the blade. 
	


2.4.2 Human Factor Neglect 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Negligent use of personal protection equipment 
	Negligent use of personal protection equipment may result in severe injuries 
	Use all the required protection equipment . 
	

	Human behaviour / incorrect behaviour 
	
	Be fully concentrated at any work . 
	Impossible to be excluded completely t. 


2.4.3 Residual Thermal Risks 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Burns, frostbites 
	A contact with the exhaust may cause burns. 
	Lat the exhaust cool down after the use . 
	


2.4.4 Noise Risks 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Hearing impairment 
	Frequent/long work with the brush cutter without ear protectors may result in hearing impairment 
	Always wear ear protectors 
	


2.4.5 Material and Other Substances Action Risk 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Contact, aspiration 
	Flue gases may be health hazardous 
	Use the machine outdoors only and have regular . 
	

	Fire or explosion 
	Fuel mix is flammable 
	Do not smoke and do not handle open fire in the course of work and refuelling . 
	


2.4.6 Vibration Risk 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Vibrations of the whole body 
	Prolonged work with the unit may produce adverse effects on the human organism owing to the vibrations . 
	Take breaks regularly . 
	


2.4.7 Other Hazards 

	Hazard 
	Description 
	Protective Action 
	Residual Hazards 

	Thrown away articles and splashes 
	Thrown away stones or pieces of wood may cause injuries to people and animals . 
	Make sure that no persons and animals stay in the area of risk and wear appropriate protection clothing , ear protectors and face shield t. 
	

	Slip, trip or a fall 
	On an uneven ground, damage may occur in result of a trip 
	Always stand safely and wear antiskid shoes. 
	


2.4.8 Disposal 

Disposal instructions are given by pictographs on the unit or packaging. For meaning of individual symbols refer to chapter “Symbols on Unit”. 

2.5 Operator Requirements

The operator shall read the instruction manual carefully before using the unit. 

2.5.1 Qualification 

No special qualification is required for use of the unit except for detailed direction by a professional. 

2.5.2 Minimum Age 

Only persons above 16 years of age are allowed to work with the unit. .Exempted from the provision is the use of the juvenile trainees if they work in the course of theirprofessional training with an aim to obtain the skill under a trainer supervision.

2.5.3 Training 

Use of the unit requires adequate lesson by a professional or the use of the manual only . Special training is not required. 

3. Technical Data

	 (Fig. 1) 
	6 in 1 – set of petrol multi-purpose tools: Driving unit 

	Engine design 
	1-cylinder 2-stroke 

	Volume 
	25.4 cc

	Maximum caoacity 
	0,75 kW/1 PS 

	Speed 
	8500 min-1 

	Fuel /tank volume 
	2-strike engine mix 1:25/0,75 l 

	Starting system 
	Recoil starter 

	Acoustic output level Measured Guaranteed 
	93 dB (A) 96 dB (A) 

	Weight : 
	4,2 kg 


	(Fig. 2) 
	6 in 1 – a set of petrol multi-purpose tools : Extension 

	Length 
	620 mm 

	Weight 
	0,5 kg 

	(Fig. 3) 
	6 in 1 a set of  petrol multi-purpose tools - pruning saw ( Oregon chain and blade) 

	Length 
	910 mm 

	Blade length 
	25 cm 

	Weight 
	1,67 kg 


	(Fig. 4) 
	6 in 1 – a set of petrol multi-purpose tools – Hedge trimmer 

	Length 
	1130 mm 

	Max. cutting length 
	400 mm 

	Max. cutting width 
	24 mm 

	Weight ca 
	2.25 kg 


	(Fig. 5) 
	6 in 1 – a set of petrol multi-purpose tools: blower

	Air velocity
	67m/s (150mph)

	Air volume 
	474 cubic m/h (279 cfm)

	Weight 
	2 kg 


	(Fig. 6) 
	6 in 1 – a set of petrol multi-purpose tools: Bush cutter

	Length 
	880 mm 

	Blade size 
	255 x 1,6 mm 

	Weight 
	1,7 kg 


	(Fig. 7) 
	6 in 1 – set of petrol multi-purpose tools: grass trimmer

	Max. cutting width 
	380 mm 

	String diameter 
	2,2 mm 

	Weight  
	1.14 kg 


	(Fig. 8) 
	6 in 1 – set of petrol multi-purpose tools: edger

	Blade 
	200mmX3mmX2mm

	Cutting depth
	50mm

	Weight  
	1.9 kg 


4. Transportation and Storage

If the set of tools 6 in 1 is to be out of operation for a longer period of time (1 month), take action as follows: 

· Drain the oil and fuel tank . 
· Remove and clean the string coil and the blade . 
· Wipe the string coil and the blade dry . 
· Wipe the external part of the se with a soft cloth soaked in fine suds . 
· Store the set at an elevated locked place outside the reach of children . 
· Sore the set at a dry place . 
· Blade, pruning saw, hedge scissors and cultivator should be packed well to prevent incisions. 

5. AssemblyandInitialOperation

5.1 Set of etrol multi-purpose tools 6 in1:DrivingUnit

Structural  Group1                      Delivered parts and accessories

[image: image52.jpg]Fig. 1-pos. 1




5.2 Set of petrol multi-purpose tools 6 in1:grass trimmer
[image: image53.jpg]Structural Group 2

Delivered parts and
accossorios

Fig. 6-pos. 1
Fig. 6-pos. 2





5.3 Set of petrol multi-purpose tools 6 in1:Bruch cutter
Structural Group 2                               Delivered parts and accessories

Fig. 7-Driving unit Fig. 6-Brush cutter
[image: image54.jpg]Fig. 7-Driving unit
Fig. 6-Bush clipper





[image: image55.jpg]



Fig. 10 
5.4 set of petrol multi-purpose tools 6 in1: blower


Fig. 11
5.5 set of petrol multi-purpose tools 6 in1: hedge trimmer
[image: image56.jpg]Structural Group 5

Delivered parts and
accessories

Fig. 1-Driving unit
Fig. 4-Hedge scissors





5.6 set of petrol multi-purpose tools 6 in1: Pruning saw
[image: image57.jpg]‘Structural Group 6

Delivered parts and
accessories

Fig. 1-Driving unit
Fig. 3-Pruning saw





5.7 set of petrol multi-purpose tools 6 in1:Extension

	Structural Group7
	Delivered parts and accessories

	[image: image58.emf]
	Fig. 2– Extension 

	Fig. 14 
	


5.8 set of petrol multi-purpose tools 6 in1:Edger
Fig. 15

5.9 Shoulder harness Assembly

Structural Group 8                                           Delivered parts and accessories

Fig. 1 – pos. 9 

5.10 Fuel information

Refuelling: 
For air-cooled 2-stroke engines, mix unleaded petrol well with a branded oil at a ratio of 1:25. Meaning that 

0.2 litre oil has to be added in 5 litres of petrol. .Tip: Always prepare 1 litre of the mix at a time because the mix turns unusable after prolonged storage. .

5.11 Initial Operation Safety Instructions

Warning ! Before putting the set in operation, check it to see tthat the condition is safe with regard to operation. Do not switch the machine on, if in doubt ! 

Keep a watch on the points shown below: 
· Cirrect assembly iof integrated tools . 
· Easy run of all the switches 
· Safe installation of the ignition plug . When the plug is loose, sparkles may be generated and the flowing mix of fuel and air may get ignited. 
· Make sure that the handle is clean to conduct the set safely. Before starting the machine, all the safety and protection devices have to be fitted correctly at places. . 

All the integrated tools must run freely .Before starting the set, make sure that the integrated tools are fitted on the driving unit correctly and that themoving parts of the machine run freely. .

Warning ! If in doubt, require assistance of an expert from the authorised servicing centre . Laic advice may 

[image: image59.jpg]



1 CAUTION! Held your hands and legs out of the reach of the integrated tool, mainly when the machine is being started. Always have your hand on the additional handle lightly . Always use the machine with the carrying strap supplied. Hld one hand on the engine handle and the other one in the axle handle. 

2 Always hold the unit at an appropriate safe distance from the body and take a stable posture. . 

3 Always use a face shield . 

4 Use the machine at daylight only. 

5 Do not use the machine in rain or on wet grass 

6 Check the machine after a stroke for possible damages and have it repaired if necessary . 

7 Do not use the machine if the protection devices are damaged or installed incorrectly . 

8 Make sure that the engine vents are free of any dirt and remnants . 

9 In the course of work procedure, make sure that nopersons or animals are standing within 10 m diameter . Switch the machine off immediately if anybody, cildren in the first place, enter the reach of machine. 
10 Using the set, stones or other parts may be thrown away and may cause serious injuries. 
11 Do not get close to moving parts (where they have cutting devices – integrated bush clipper) 

12 Before using the tool, the work area must be cleared from any stone, branches and other solid material 

13 After the cutting fibre is extended, maximum attention is required (brush cutter) . Having accomplished the procedure reclaim the correct working position before the machine is out in operation again. . 

14 Do not use metal cutting coils. . 

15 Pay attention to the machine running for another five seconds after relieving the switch. . 

6. Operation 
6.1 Starting 6 in1 petrol multi-purpose tools :
Start the set of tools only after an integrated piece has been connected ! 
[image: image60.jpg]Put the operating switch at position 1.

1S

Fig. 17

Put the starting carburetor lever in position Start,

| Fig.18

Push the primary pump 3-5 times.

Fig. 19

« Push unlocking knob and accelerate
« Pushthe half throttle lock in at this position
(cold)





[image: image61.jpg]Start the machine pushing the starter cable in a
controlled manner .

Doing that, push device 2 to the ground with
yiur hand .





[image: image62.jpg]Now, control the gas lever o that the machine
runs evenly for it ip reach full speed. .

Ason as the engine start running, the choke
may be returned in the original position .





6.2 Blade Removal:
	Structural group1
	Required structural
	Delivered parts and
	

	
	group . 
	accessories . 
	

	
	Structural group 3 
	Fig. 1-pos. 1 
	

	
	
	Fig. 1-5 
	

	
	
	Removed: 
	

	Fig. 23 
	
	
	


	Structuralgroup2
	Requiredstructural
	Deliveredpartsand
	

	
	group . 
	accessories . 
	

	
	Structural group 3 
	Fig. 3 – pos. 15 Fig. 3 – pos. 17 + 18 
	

	
	
	Removed Fig. 3 – pos. 10 Fig. 3 – pos. 19 Fig. 3 – pos. 20 Fig. 3 – pos. 21 Fig. 3 – pos. 23 
	

	Fig. 24 
	
	
	


6.3 Ignition plug replacement:
[image: image63.jpg]Pull the ignifion plug cover off, screw the plug out using
the plug wrench and replace it
(Fig. 3/pos. 17+18)





6.4 Operator Safety Instructions

6.4.1 grass trimmer and Brush cutter Operation Instructions 

To turn the machine off, the switch has to be in „STOP“ position . 

· When mowing, mind the specific regulations of a country or specific communal regulations . 
· Do not mow at times of generally recognised rest . 
· Any hard objects – stones, metal parts etc. – have to be removed. They could be thrown away and cause prasonal or material damage. 
· Working in high shrubs or bushes, the working height must be 15 cm as a minimu not to expose animals, such as hedgehogs at any risk. 

Grass Mowing 
-Working on grass areas, hold the scythe at an angle of ca 30°and make semicircle turns to the left and right. 

The best results are achieved with grass 15 cm high. If it is higher, it is recommended to repeat mowing several times. 

CAUTION! When the machine is running never put the mowing head on the ground. 
If the mowing output decreases 
1.) Hold the running machine over the grass and knock the ground several times with the mowing head The fibre will get prolonged in thus way. 2.) The cutter enclosed in the protection shield will cut the fibre to the required length. 

Caution! The fibre remnant may be thrown off and cause an injury. 
If the machine is vibrating: 
Clean it or, where necessary, and remove the grass stuck on the mowing head and protection shield. 

1 Hold the clipper form and safely with both the hands. 

2 Mow the grass and weed only, low shrubbery may be cut with a thick blade. 

3 Work in a regardful manner and do not threaten anybody while mowing. Work quietly and considerately! 

4 Work only in conditions of sufficient visibility and sufficient light ! 

5 Watch the cutting head ! 

6 Never mow/cut above the height of your shoulders ! 

7 Never replace the plastic cord with steel wire – risk of injury and destruction! 

8 Never work standing on a ladder ! 

Work on strong and stable bases ! 

· Use the unit after you have read the operating instructions carefully . 
· Observe all the safety instruction in the manual . 
· Be responsible to the others . 

	6.4.2 
	Pruning Saw Operation Instructions 

	
	Anoid a backflash. Back lashes may cause severe injuries . Do not use the pruning saw standing on a ladder or another aid in unstable position . You may loose control over the saw and suffer severe injuries. If heavies/larger machines are required due to the circumstances, request support from professionals. 

	
	Beware of the conditions you are working in . An engine unit will produce flue gases as soo as the engine is running . The gases may be invisible and odourless. Therefore, never work with the unit in closed and poorly ventilated rooms. Provide for sufficient lighting . Mind safe standing position when working in wet, snow, ice, slopes and uneven ground. . 

	• • 
	Do not use the unit until you have read instruction manual carefully . Observe all the safety regulations show in the manual. . Be responsible to the others . 

	
	Caution: 1.Beware of falling branches.
         2.Must assemble the extension .


6.4.3 blower Operation Instructions 
· While blowing debris, hold the unit with the muffler side facing away from your body and clothes (see illustration above).
· To reduce the risk of hearing lossassociated with sound level(s), hearing protection is required.
· To reduce the risk of injury associated with contacting rotating parts, stop the engine before installing or removing attachments. Do not operate without guard(s) in place.
· To reduce noise levels, limit the number of pieces of equipment used at any one time.
· To reduce noise levels, operate power blowers at the lowest possible throttle speed to do the job.
· Use rakes and brooms to loosen debris before blowing.
· In dusty conditions, slightly dampen surfaces or use a mister attachment when water is available.
· Conserve water by using power blowers instead of hoses for many lawn and garden applications, including areas such as gutters,screens, patios, grills, porches, and gardens.
· Watch out for children, pets, open windows,or freshly washed cars. Blow debris away safely.
· Use the full blower nozzle extension so the air stream can work close to the ground.
· After using blowers and other equipment,

CLEAN UP! Dispose of debris in trash receptacles.

6.4.4 Hedge trimmer Operating Instructions 

· Use the unit only after you have read the instruction manual carefully . 
· Observe all the safety instructions contained in the manual . 
· Hedge scissors may be used not only on hedges but also to cut shrubs. . 
· The best cutting output may be obtained when the scissors are conducted at an approximate angle of 15°to the hedge . 
· To obtain an even height of the hedgerow, it is recommended to have a conducting cord stretched along the hedge. Any branched beyond the line will be cut off. . 
· Side surface area of the hedge will be trimmed by arched movements from down upwards. 
Caution:  Must assemble the extension .
6.4.5  edger Operating Instructions 

· Allow the engine to warm up for one minute before you begin edging.
· Increase the engine speed before placing the blade in the cut. For best operation, run the engine at full throttle while cutting.
· Keep you edging path straight by aligning the blade guide rib on the rear of the blade guard with the edge of the sidewalk. Keep all wheels flat on the walkway.
· Always work going away from people and solid objects such as walls, large stones,trees, automobiles, etc.
· Be careful when edging near trees or valuable plants. The high speed metal blademay cut roots and cause damage to theplants.
· If the blade stalls, immediately move the unit rearward slightly to allow the blade to restart. If the blade continues to stall, stop the engine, disconnect the spark plug, and inspect for blockage or damage.
· Always keep the blade area clean. Stop the engine, make sure the blade has completely stopped turning, and disconnect the spark plug before cleaning.

6.5 Step by Step Instructions

· Remove any foreign objects from the ground to be treated. 
· Put appropriate protection clothing on . 
· Start the unit according to the description in chapter “Operation”. 
· Observe all the relevant safety instructions . 
· Clean the unit after every use and check all the integrated tools and protection devices for damages. 

7. TroublesCausesTroubleshooting 

	Trouble 
	Cause 
	Troubleshooting 

	The engine will not start 
	1. Empty tank 2. The engine is flooded 3. Fouled plug, (the carbon residues on electrodes) too big distance between the electrodes. 4. Fouled carburettor or carburettor jets. 5. Damaged plug connection or ignition cable. 6. Ignition spark missing 
	1. fill up the tank 2. Close the throttle, try starting several times, remove the ignition plug if necessary and dry it. 3. Clean the plug, check the thermal value and replace it if necessary Set 0.6-0.7 mm 4. Clean the carburettor blow it off 5. Replace 6. Check short-circuit knob, the cable ignition module and replace if necessary. Adjust the gap (0,3-0,4 mm). 

	When idling, the engine is too fast 
	1. Cold engine 
	11. Heat up slowly, turn down the choke if necessary . 

	The engine will not reach the maximum output 
	1. Fouled plug, (the carbon residues on electrodes) too big distance between the electrodes. 2. Fouled carburettor or carburettor jets. 3. Fouled air filter 4. Exhaust, exhaust pipe 5. The packing ring in the crankcase leaking Worn cylinders, pistons , piston rings 6. Poor fuel mix (too much oil 7. Poor ignition 
	1. Clean the plug, check the thermal value and replace it if necessary Set 0.6-0.7 mm 5. Clean the carburettor blow it off 2. Clean 3. Remove the exhaust pipe, clean and remove carbon residues. 4. Replace 5. Replace the cylinder, piston or piston rings, grind cylinder and piston, install 6. fill up according to instructions 7. Set a gap in the ignition module (0.3-0.4 mm) 

	The engine will not stop 
	1. The plug is pale grey with marks of white-hot (melting drops)) 2. Carbon residue in the ignition area. 
	1. Fit in the plug of higher thermal value . 2. Clean the cylinder head, piston bottom and cylinder the engine block 


[image: image64.jpg]



8. InspectionsandMaintenance

[image: image65.jpg]



CAUTION!The instructions below shall apply to regular conditions of use. In severe conditions and longer periods of workdays it is necessary to shorten the intervals as appropriate. 
After having accomplished the job, always check 
The entire machine and carry out the cleaning 

Regular check and treatment: 
· Check the air filter (clean it up if necessary) 
· Check the ignition plug (replace if necessary) 
· The function and quantity of the plastic cord. 

[image: image66.jpg]


Always swith the driving unit off when doing maintenance or a repair 
[image: image67.jpg]



8.1 Inspections and Maintenance Safety Instructions

[image: image68.jpg]



Only a unit maintained and cared for on a regular basis may be a serviceable aid. Insufficient maintenance and care may result in emergencies and unforeseen accidents. 

Only authorised servicing centres may do repairs requiring special orifessional knowledge . Unprofessional action may cause damage of the machine or put your safety at risk. 

[image: image69.jpg]



8.2 Inspections and Maintenance Schedule

[image: image70.jpg]



	Time Interval 
	Description 
	Othe Details if necessary 

	Before every start of work 
	• Check the gas lever 
	

	Before every start of work 
	• Check on the entire machine visually j 
	

	Before every start of work 
	•Zkontrolujte usazení daného vestavného přístroje 
	

	After every use 
	•Zkontrolujte celý stroj a proveďte nezbytné čištění. 
	


9.SpareParts

[image: image71.jpg]



Claims and orders of spare parts will be attended to fast and smoothly. Use the form at 

www.hszbjx.com
1





2





3





lock screw


blade


adjusting wheel





2





1








1

_1238372964.psd

_1238372965.unknown

_1238372962.psd

_1238372963.psd

_1238372961.psd

